HorschConnect Telematics API

API Reference Documentation

Version 1.2  •  2026-05-26

https://telematics-api.horsch.com

Contact: portal@horsch.com

Swagger: https://telematics-api.horsch.com/swagger/index.html

Docs: https://telematics-doc.horsch.com/en/api

# Document Version History

| Version | Date | Author | Changes |
| --- | --- | --- | --- |
| 1.2 | 2026-05-26 | HORRSCH Maschinen SE & Co. KG | Added v2.0 API support with new telemetry data structure; side-by-side comparison; migration guide |
| 1.1 | – | HORRSCH Maschinen SE & Co. KG | Initial API documentation with v1.1 endpoints |

# Table of Contents

1.   Overview

2.   Authentication

3.   API Versioning

4.   API Resources

4.1   Machines Resource – Machine Overview

4.2   Telemetry Resource – Machine Telemetry Data

4.3   Errors Resource – Machine Error Codes

5.   Quick Start Guide

6.   v2.0 Changes and Migration

6.1   Overview of Changes

6.2   Key Differences: v1.1 vs v2.0

6.3   Side-by-Side Comparison

6.4   Designator Format Explained

6.5   Common DDI Codes

6.6   Migration Guide (Developers + Support)

7.   Data Models

7.1   Telemetry Data Fields by Machine Type

7.2   ISO 11783 DDI Reference (v2.0)

8.   Error Handling

8.1   HTTP Status Codes

8.2   Error Response Format

8.3   Rate Limiting

9.   Best Practices

10.  Support & Contact

# 1. Overview

The HorschConnect Telematics API provides programmatic access to HORSCH agricultural machinery data. The API exposes three main resources:

Machines – Overview and last position of connected machines. Telemetry – Per-second agronomic data (application rates, GPS, working status). Errors – Machine error codes with severity and location.

### Supported Machine Types

| Property | Type | Description |
| --- | --- | --- |
| Seeding | model | Pronto, Sprinter, Express, and similar drill machines. |
| Planting | model | Maestro series planters. |
| CropCare | model | Leeb sprayers. |

# 2. Authentication

The API uses API Key authentication via the X-API-Key request header. Every request must include a valid API key.

### Obtaining an API Key

Contact HORSCH support at portal@horsch.com to request API access credentials.

### Using the API Key

Include the key in every request header:

X-API-Key: your-api-key-here

### Example Authentication

curl -X GET \

"https://telematics-api.horsch.com/v1.1/machines" \

-H "X-API-Key: your-api-key-here"

Security: Keep your API key confidential. Do not expose it in client-side code or public repositories. Contact portal@horsch.com to revoke a compromised key.

# 3. API Versioning

The API supports multiple concurrent versions to ensure backward compatibility. Specify the version as the first path segment of every request URL.

### Available Versions

| Version | Base Path | Status | Notes |
| --- | --- | --- | --- |
| v1.1 | /v1.1/ | Active | Original telemetry structure with boom-based data. Fully supported. |
| v2.0 | /v2.0/ | Active | New telemetry structure with designator-based data (ISO 11783 DDI). Recommended for new integrations. |

### Version Selection

Specify the version in the URL path:

https://telematics-api.horsch.com/v1.1/{resource}

https://telematics-api.horsch.com/v2.0/{resource}

Note: The Machines and Errors endpoints are identical across both versions. Only the Telemetry endpoint returns different data structures. See Section 6 for migration details.

# 4. API Resources

## 4.1 Machines Resource – Machine Overview

Retrieve a list of all machines accessible with your API key, including the last known GPS position of each machine.

Version note: The machine list endpoint is identical across v1.1 and v2.0.

| GET | /v1.1/machines |
| --- | --- |

| GET | /v2.0/machines |
| --- | --- |

### Query Parameters

| Parameter | Type | Required | Description |
| --- | --- | --- | --- |
| cursor | string | No | Pagination cursor for retrieving the next page of results. Omit on first request. |

### Response Fields

| Property | Type | Description |
| --- | --- | --- |
| machines[] | array | Array of machine objects. |
| chassisNumber | string | Unique HORSCH serial number (e.g. 24123456). |
| name | string | Machine display name (e.g. Pronto 7.2 DC M22 SP05). |
| model | string | Machine model category: Seeding, Planting, or CropCare. |
| lastPosition | object | Last recorded GPS position object. |
| lastPosition.latitude | number | Latitude coordinate (WGS 84). |
| lastPosition.longitude | number | Longitude coordinate (WGS 84). |
| lastPosition.timestamp | string (ISO 8601) | UTC timestamp of last recorded position. |
| nextCursor | string | Pagination cursor for the next page. null if no further results. |

### Example Request

curl -X GET \

"https://telematics-api.horsch.com/v1.1/machines" \

-H "X-API-Key: your-api-key-here"

### Example Response – 200 OK

{

"machines": [

{

"chassisNumber": "24123456",

"name": "Pronto 7.2 DC M22 SP05",

"model": "Seeding",

"lastPosition": {

"latitude": 54.084530,

"longitude": 10.405337,

"timestamp": "2024-07-30T12:01:41.930Z"

}

},

{

"chassisNumber": "25123456",

"name": "Leeb 6.280 PT",

"model": "CropCare",

"lastPosition": {

"latitude": 53.053155,

"longitude": 76.907558,

"timestamp": "2024-06-08T18:01:42.040Z"

}

}

],

"nextCursor": "MjUxODE0OTY="

}

### Pagination Example

# First page

GET /v1.1/machines

# Next page (URL-encode the cursor value)

GET /v1.1/machines?cursor=MjUxODE0OTY%3D

## 4.2 Telemetry Resource – Machine Telemetry Data

Returns per-second telemetry data for a specific machine in GeoJSON FeatureCollection format. The response structure differs between v1.1 and v2.0 for Seeding/Planting machines. See Section 6 for a detailed comparison.

Important: The data structure for Seeding/Planting machines differs between v1.1 and v2.0. CropCare/Sprayer structure is unchanged across versions.

| GET | /v1.1/machines/{chassisNumber}/telemetry |
| --- | --- |

| GET | /v2.0/machines/{chassisNumber}/telemetry |
| --- | --- |

### Path Parameters

| Parameter | Type | Required | Description |
| --- | --- | --- | --- |
| chassisNumber | string | Yes | Unique HORSCH serial number of the machine (e.g. 24123456). |

### Query Parameters

| Parameter | Type | Required | Description |
| --- | --- | --- | --- |
| from | string (ISO 8601) | No | Start timestamp for telemetry data (e.g. 2024-07-29T00:00:00Z). |
| to | string (ISO 8601) | No | End timestamp for telemetry data. |
| cursor | string | No | Pagination cursor from a previous response. |

### Common Response Fields – All Versions

| Property | Type | Description |
| --- | --- | --- |
| machine.chassisNumber | string | Machine chassis number. |
| machine.name | string | Machine name. |
| machine.model | string | Machine model category. |
| nextCursor | string | Pagination cursor for the next page. |
| geoJson.type | string | Always "FeatureCollection". |
| geoJson.features | array | Array of per-second telemetry data points. |
| geometry.type | string | Always "Point". |
| geometry.coordinates | array | GPS position as [longitude, latitude]. |
| properties.recordedAt | string (ISO 8601) | Timestamp recorded on the machine. |
| properties.storedAt | string (ISO 8601) | Timestamp stored in the cloud. |
| properties.velocity | number / object | Speed in km/h. Plain number in v1.1 (Seeding); {value, unit} object in v1.1 (CropCare) and all v2.0. |
| properties.course | number / object | Course/heading in degrees. Same versioning pattern as velocity. |
| properties.working | boolean | True if machine is currently in working state. |
| properties.hoppers[] | array | Hopper data array (Seeding/Planting machines). |
| properties.booms[] | array | Boom-level application data. Structure differs by version (see below). |

### v1.1 Telemetry – Seeding / Planting

Boom data uses integer boom IDs with named fields for setpoint, actual rate, crop type, and variation coefficient.

| Property | Type | Description |
| --- | --- | --- |
| booms[].boom | integer | Boom number (1-based index). |
| booms[].setpoint | number | Target application rate (plain number, unit depends on machine config). |
| booms[].actual | number | Actual measured application rate. |
| booms[].cropType | string | Crop type for this boom (e.g. Wheat). |
| booms[].variationCoefficient | number | Seed distribution uniformity coefficient (CV) in %. |

Example Response – v1.1 Seeding/Planting (200 OK)

{

"machine": { "chassisNumber": "21661283", "name": "Pronto 7,2 DC M22 SP05", "model": "Seeding" },

"nextCursor": "MjAyNC0wNy0zMFQxMjowMTo0MS45MzAwMDAwWg==",

"geoJson": {

"type": "FeatureCollection",

"features": [{

"type": "Feature",

"geometry": { "type": "Point", "coordinates": [10.405337, 54.084530] },

"properties": {

"recordedAt": "2024-07-30T12:00:01.148Z",

"storedAt": "2024-07-30T12:01:26.321Z",

"velocity": 1.975,

"course": 318.87,

"working": false,

"hoppers": [],

"booms": [{

"boom": 1,

"setpoint": 150.0,

"actual": 159.9,

"cropType": "Wheat",

"variationCoefficient": 10.1

}]

}

}]

}

}

### v1.1 Telemetry – CropCare / Sprayer

Sprayer data is contained in a main object (since LSB Version V1.15.1) with named fields for all application parameters.

| Property | Type | Description |
| --- | --- | --- |
| properties.lifetimeTotalArea | object | Lifetime area counter: { value, unit: "ha" }. |
| main.setpoint | object | Target application rate: { value, unit: "l/ha" }. |
| main.actual | object | Actual application rate: { value, unit: "l/ha" }. |
| main.actualVolumeContent | object | Current tank fill level: { value, unit: "l" }. |
| main.actualWorkingHeight | object | Boom height above target: { value, unit: "cm" }. |
| main.setpointProductPressure | object | Target spray pressure: { value, unit: "bar" }. |
| main.actualProductPressure | object | Actual spray pressure: { value, unit: "bar" }. |

Example Response – v1.1 CropCare / Sprayer (200 OK)

{

"machine": { "chassisNumber": "25123456", "name": "Leeb 6.280 PT", "model": "CropCare" },

"nextCursor": "MjAyNC0wNi0wOFQxODowMTo0Mi4wNDAwMDAwWg==",

"geoJson": {

"type": "FeatureCollection",

"features": [{

"type": "Feature",

"geometry": { "type": "Point", "coordinates": [76.907558, 53.053155] },

"properties": {

"recordedAt": "2024-06-08T18:00:01.228Z",

"velocity": { "value": 25.24, "unit": "km/h" },

"course": { "value": 359.94, "unit": "degrees" },

"lifetimeTotalArea": { "value": 21288.98, "unit": "ha" },

"working": true,

"hoppers": [], "booms": [],

"main": {

"actualVolumeContent": { "value": 5190, "unit": "l" },

"setpoint": { "value": 70, "unit": "l/ha" },

"actual": { "value": 70, "unit": "l/ha" },

"actualWorkingHeight": { "value": 60, "unit": "cm" },

"setpointProductPressure": { "value": 6, "unit": "bar" },

"actualProductPressure": { "value": 6, "unit": "bar" }

}

}

}]

}

}

### v2.0 Telemetry – Seeding / Planting  —  NEW

Key change: The boom integer key is replaced by a designator string that encodes boom, section, and row. Data values use ISO 11783 DDI (Data Dictionary Identifier) key-value pairs instead of named fields. A new device object provides machine-level data.

| Property | Type | Description |
| --- | --- | --- |
| booms[].designator | string | Combined boom/section/row identifier (e.g. "B01¦01 00¦00"). See Section 6.4. |
| booms[].values[] | array | Array of DDI measurement objects: { ddi: integer, value: number }. |
| booms[].values[].ddi | integer | ISO 11783 Data Dictionary Identifier. See Section 6.5 for common codes. |
| booms[].values[].value | number | Raw value in base unit (apply scaling factor to convert – see Section 6.5). |
| device.designator | string | Always "Implement" for machine-level data. |
| device.values[] | array | DDI measurements at machine level (e.g. DDI 271 = lifetime area in m²). |

Example Response – v2.0 Seeding/Planting (200 OK)

{

"machine": { "chassisNumber": "21661283", "name": "Pronto 7,2 DC M22 SP05", "model": "Seeding" },

"nextCursor": "MjAyNC0wNy0zMFQxMjowMTo0MS45MzAwMDAwWg==",

"geoJson": {

"type": "FeatureCollection",

"features": [{

"type": "Feature",

"geometry": { "type": "Point", "coordinates": [10.405337, 54.084530] },

"properties": {

"recordedAt": "2024-07-30T12:00:01.148Z",

"velocity": 1.975, "course": 318.87, "working": false,

"hoppers": [],

"booms": [

{

"designator": "B01|01 00|00",

"values": [

{ "ddi": 6, "value": 1500 },

{ "ddi": 7, "value": 1599 },

{ "ddi": 58382, "value": 101 }

]

},

{

"designator": "B01|02 01|01",

"values": [

{ "ddi": 6, "value": 1300 },

{ "ddi": 7, "value": 0 },

{ "ddi": 58382, "value": 101 }

]

}

],

"device": {

"designator": "Implement",

"values": [ { "ddi": 271, "value": 114462 } ]

}

}

}]

}

}

### v2.0 Telemetry – CropCare / Sprayer  —  Unchanged

No changes in v2.0: The CropCare/Sprayer telemetry structure is identical between v1.1 and v2.0. No migration is required for sprayer integrations.

## 4.3 Errors Resource – Machine Error Codes

Retrieve machine error codes with severity levels and GPS location information. The error endpoint structure is identical across both API versions.

Version note: The error endpoint structure is identical across v1.1 and v2.0.

| GET | /v1.1/machines/{chassisNumber}/errors |
| --- | --- |

| GET | /v2.0/machines/{chassisNumber}/errors |
| --- | --- |

### Path Parameters

| Parameter | Type | Required | Description |
| --- | --- | --- | --- |
| chassisNumber | string | Yes | Unique HORSCH serial number of the machine (e.g. 24123456). |

### Query Parameters

| Parameter | Type | Required | Description |
| --- | --- | --- | --- |
| from | string (ISO 8601) | No | Start timestamp for error data. |
| to | string (ISO 8601) | No | End timestamp for error data. |
| cursor | string | No | Pagination cursor from a previous response. |

### Response Fields

| Property | Type | Description |
| --- | --- | --- |
| machine.chassisNumber | string | Unique HORSCH serial number. |
| machine.name | string | Machine name. |
| machine.model | string | Machine model category. |
| nextCursor | string | Pagination cursor for the next page. null if no more results. |
| errors[] | array | Array of error event objects. |
| errors[].errorCode | string | HORSCH error code (e.g. E0042). |
| errors[].description | string | Human-readable error description. |
| errors[].severity | string | Severity level: Info, Warning, or Critical. |
| errors[].timestamp | string (ISO 8601) | UTC timestamp when the error occurred. |
| errors[].location | object | GPS coordinates where the error occurred. |
| errors[].location.latitude | number | Latitude (WGS 84). |
| errors[].location.longitude | number | Longitude (WGS 84). |

### Severity Levels

| Severity | Meaning | Recommended Action |
| --- | --- | --- |
| Info | Informational message | No action required. |
| Warning | Potential issue | Monitor the condition. No immediate action required. |
| Critical | Machine operation affected | Immediate attention required. Check machine before continuing operation. |

### Example Request

curl -X GET \

"https://telematics-api.horsch.com/v1.1/machines/24123456/errors" \

-H "X-API-Key: your-api-key-here"

### Example Response – 200 OK

{

"machine": {

"chassisNumber": "24123456",

"name": "Pronto 7.2 DC M22 SP05",

"model": "Seeding"

},

"errors": [

{

"errorCode": "E0042",

"description": "Seed metering unit 1 blockage detected",

"severity": "Critical",

"timestamp": "2024-07-30T14:23:15.000Z",

"location": { "latitude": 54.084530, "longitude": 10.405337 }

},

{

"errorCode": "W0103",

"description": "Low seed level in hopper 2",

"severity": "Warning",

"timestamp": "2024-07-30T13:45:22.000Z",

"location": { "latitude": 54.083215, "longitude": 10.404128 }

}

],

"nextCursor": null

}

# 5. Quick Start Guide

Follow these steps to make your first successful API call.

### Step 1 – Obtain an API Key

Contact portal@horsch.com to request API credentials.

### Step 2 – Test Authentication

curl -X GET \

"https://telematics-api.horsch.com/v1.1/machines" \

-H "X-API-Key: your-api-key-here"

### Step 3 – List Available Machines

A successful 200 response returns your machine fleet. Note the chassisNumber values for use in subsequent calls.

### Step 4 – Retrieve Telemetry Data

Use the chassis number from Step 3. For new integrations, use v2.0:

curl -X GET \

"https://telematics-api.horsch.com/v2.0/machines/24123456/telemetry" \

-H "X-API-Key: your-api-key-here"

### Step 5 – Check for Errors

curl -X GET \

"https://telematics-api.horsch.com/v1.1/machines/24123456/errors" \

-H "X-API-Key: your-api-key-here"

Tip: Use the Swagger UI to explore and test all endpoints interactively: https://telematics-api.horsch.com/swagger/index.html

# 6. v2.0 Changes and Migration

Version 2.0 introduces a new telemetry data structure for Seeding and Planting machines based on the ISO 11783 DDI standard. This section provides everything needed for a successful migration.

## 6.1 Overview of Changes

The core change in v2.0 is the replacement of named boom fields with standardized ISO 11783 DDI (Data Dictionary Identifier) key-value pairs. This enables more granular data at the metering-unit level and better interoperability with other ISO 11783-compliant systems.

Seeding / Planting: Significant structural change. booms[] array now uses designator strings instead of boom integers, and values are DDI-indexed instead of named fields. Migration required. CropCare / Sprayer: No changes. The main object structure is identical in both versions. No migration required.

## 6.2 Key Differences: v1.1 vs v2.0

### Seeding / Planting Machines

| Aspect | v1.1 | v2.0 |
| --- | --- | --- |
| Boom Identification | boom (integer) | designator (string) |
| Data Format | Named fields (setpoint, actual, ...) | DDI-based key-value pairs |
| Granularity | Boom level only | Boom + Metering unit + Row combined |
| Designator Format | N/A | "B01¦01 00¦00" (Boom¦Section Row1¦Row2) |
| Data Values | Direct field access | Array of { ddi, value } objects |
| Machine-level data | Not available | New device object with DDI values |

### CropCare / Sprayer Machines

| Aspect | v1.1 | v2.0 |
| --- | --- | --- |
| Structure | main object with named fields | Unchanged – main object remains |
| Notes | LSB Version >= V1.15.1 uses main instead of boom array | Same behavior in v2.0 |

## 6.3 Side-by-Side Comparison

### Boom Data: v1.1 vs v2.0 (Seeding/Planting)

| v1.1 Format | v2.0 Format (NEW) |
| --- | --- |
| {"booms": [ { "boom": 1, "setpoint": 150.0, "actual": 159.9, "cropType": "Wheat", "variationCoefficient": 10.1 } ]} | {"booms": [ { "designator": "B01¦01 00¦00", "values": [ { "ddi": 6, "value": 1500 }, { "ddi": 7, "value": 1599 }, { "ddi": 58382, "value": 101 } ] } ],"device": { "designator": "Implement", "values": [{ "ddi": 271, "value": 114462 }] } |

## 6.4 Designator Format Explained

The designator string encodes boom, section, and row information in a single identifier following this pattern:

"B{boom}|{section} {row1}|{row2}"

### Designator Examples

| Designator | Boom | Section | Row | Meaning |
| --- | --- | --- | --- | --- |
| "B01¦01 00¦00" | 1 | 1 | 0 | Boom 1, Section 1 |
| "B01¦02 01¦01" | 1 | 2 | 1 | Boom 1, Section 2, Row 1 |
| "Implement" | – | – | – | Device-level machine data |

## 6.5 Common DDI Codes

Values are transmitted in base units and must be scaled for display. The table below shows the most common DDIs used in HorschConnect telemetry:

| DDI | Description | Transmitted Unit | Scale Factor | Display Example |
| --- | --- | --- | --- | --- |
| 6 | Setpoint Volume per Area | 0.001 l/ha | ÷ 1000 | 1500 → 1.5 l/ha |
| 7 | Actual Volume per Area | 0.001 l/ha | ÷ 1000 | 1599 → 1.599 l/ha |
| 58382 | Variation Coefficient | 0.1 % | ÷ 10 | 101 → 10.1 % |
| 271 | Lifetime Total Area | 1 m² | None (convert ÷ 10000 for ha) | 114462 m² = 11.45 ha |

Common mistake: Forgetting to apply scaling factors when reading DDI values. Always divide DDI 6 and 7 by 1000 for l/ha, and DDI 58382 by 10 for %.

## 6.6 Migration Guide

### For Software Developers

Step 1: Update endpoint URLs

// Old (v1.1)

const url = `https://telematics-api.horsch.com/v1.1/machines/${id}/telemetry`;

// New (v2.0)

const url = `https://telematics-api.horsch.com/v2.0/machines/${id}/telemetry`;

Step 2: Update parsing logic for Seeding/Planting machines

// Old (v1.1) – named fields

feature.properties.booms.forEach(boom => {

const boomNumber = boom.boom;

const setpoint   = boom.setpoint;

const actual     = boom.actual;

const cv         = boom.variationCoefficient;

});

// New (v2.0) – DDI-based values

feature.properties.booms.forEach(boom => {

const designator = boom.designator;

const setpoint   = boom.values.find(v => v.ddi === 6)?.value / 1000;  // l/ha

const actual     = boom.values.find(v => v.ddi === 7)?.value / 1000;  // l/ha

const cv         = boom.values.find(v => v.ddi === 58382)?.value / 10; // %

});

// New in v2.0: machine-level data from device object

const device      = feature.properties.device;

const lifetimeM2  = device.values.find(v => v.ddi === 271)?.value;       // m²

Step 3: CropCare/Sprayer – no changes required

// Identical in both v1.1 and v2.0

const main     = feature.properties.main;

const setpoint = main.setpoint.value;

const actual   = main.actual.value;

const pressure = main.actualProductPressure.value;

### Migration Timeline Recommendation

| Phase | Activity | Prerequisite |
| --- | --- | --- |
| Phase 1 | Test v2.0 endpoints in development environment | API key with v2.0 access |
| Phase 2 | Run v1.1 and v2.0 in parallel for validation | Completed Phase 1 |
| Phase 3 | Migrate production systems to v2.0 | Successful parallel validation |
| Phase 4 | Deprecate v1.1 usage (timeline TBD) | Completed Phase 3 |

### For 3rd Level Support Personnel

Use the following checklist when troubleshooting telemetry data issues:

Troubleshooting Checklist

| # | Check | Symptom | Likely Cause | Action |
| --- | --- | --- | --- | --- |
| 1 | API version in URL | Missing boom field in response | Client calls v2.0 but expects v1.1 | Verify client uses correct version path: /v1.1/ or /v2.0/ |
| 2 | API version in URL | Missing designator field | Client calls v1.1 but expects v2.0 | Update client to call /v2.0/ endpoint |
| 3 | DDI scaling | Application rate values appear 1000x too high | DDI 6/7 not divided by 1000 | Ensure scaling: ddi_value / 1000 for l/ha |
| 4 | DDI scaling | CV values appear 10x too high | DDI 58382 not divided by 10 | Ensure scaling: ddi_value / 10 for % |
| 5 | Firmware version (CropCare) | Sprayer data missing from main object | LSB version < V1.15.1 | Update machine firmware to >= V1.15.1 |

# 7. Data Models

## 7.1 Telemetry Data Fields by Machine Type

### Seeding / Planting Machines (Maestro)

| Data Point | Notes |
| --- | --- |
| Chassis number | Unique HORSCH serial number |
| Machine name and model | Seeding or Planting |
| GPS position (longitude, latitude) | WGS 84 |
| Speed (velocity) | km/h |
| Heading (course) | Degrees |
| Timestamp | Recorded on machine (recordedAt) |
| Lifetime area counter | Hectares (ha) |
| Working status | Boolean |
| Setpoint application rate | Volume, mass, or count per ha – by boom/designator |
| Actual application rate | Volume, mass, or count per ha – by boom/designator |
| Crop type | By hopper (Maestro only) |
| Variation coefficient (CV) | Seed distribution uniformity |
| Skips | Seed skips detected (%) |
| Doubles | Seed doubles detected (%) |

### CropCare Machines (Sprayer)

| Data Point | Notes |
| --- | --- |
| All common fields | Same as Seeding/Planting above |
| Lifetime total area | Hectares |
| Actual volume content | Current tank fill in litres |
| Setpoint application rate | l/ha |
| Actual application rate | l/ha |
| Actual working height | Boom height above target in cm |
| Setpoint product pressure | bar |
| Actual product pressure | bar |

## 7.2 ISO 11783 DDI Reference (v2.0)

Common DDI codes used in HorschConnect. For a complete reference consult the ISO 11783-11 standard.

| DDI | Name | Unit | Resolution | Range |
| --- | --- | --- | --- | --- |
| 6 | Setpoint Volume per Area Application Rate | l/ha | 0.001 | 0 – 2,147,483.647 |
| 7 | Actual Volume per Area Application Rate | l/ha | 0.001 | 0 – 2,147,483.647 |
| 271 | Lifetime Total Area | m² | 1 | 0 – 4,294,967,295 |
| 58382 | Variation Coefficient | % | 0.1 | 0 – 100.0 |

# 8. Error Handling

## 8.1 HTTP Status Codes

| Code | Status | Meaning | Action |
| --- | --- | --- | --- |
| 200 | OK | Request successful | Process response data. |
| 400 | Bad Request | Invalid parameters | Check query parameters and request format. |
| 401 | Unauthorized | Invalid or missing API key | Verify X-API-Key header is present and correct. |
| 403 | Forbidden | Insufficient permissions | Contact portal@horsch.com for access. |
| 404 | Not Found | Resource not found | Verify chassis number and endpoint URL. |
| 429 | Too Many Requests | Rate limit exceeded | Implement exponential backoff and retry. |
| 500 | Internal Server Error | Server error | Retry after delay. Contact support if it persists. |
| 503 | Service Unavailable | Service temporarily down | Retry with exponential backoff. |

## 8.2 Error Response Format

All error responses follow this JSON structure:

{

"error": {

"code": "INVALID_CHASSIS_NUMBER",

"message": "The specified chassis number does not exist or you do not have access to it.",

"timestamp": "2024-07-30T15:30:00Z"

}

}

## 8.3 Rate Limiting

Rate limits protect service availability for all users. Contact support for your specific limits.

### Rate Limit Response Headers

X-RateLimit-Limit: 100

X-RateLimit-Remaining: 95

X-RateLimit-Reset: 1627654800

Contact support: Specific rate limit values (requests per minute, concurrent connections) are provided upon API key issuance. Contact portal@horsch.com for details.

# 9. Best Practices

## 9.1 Pagination

Always implement cursor-based pagination. Never assume all data is returned in a single response.

# Python example: fetch all machines

def fetch_all_machines(api_key):

machines, cursor = [], None

while True:

url = "https://telematics-api.horsch.com/v1.1/machines"

if cursor:

url += f"?cursor={cursor}"

data = requests.get(url, headers={"X-API-Key": api_key}).json()

machines.extend(data["machines"])

cursor = data.get("nextCursor")

if not cursor:

break

return machines

## 9.2 Time Range Filtering

Use from and to parameters to limit data volume and improve response times.

# Get last 24 hours of telemetry (v2.0)

curl -X GET \

"https://telematics-api.horsch.com/v2.0/machines/24123456/telemetry

?from=2024-07-29T00:00:00Z&to=2024-07-30T00:00:00Z" \

-H "X-API-Key: your-api-key-here"

## 9.3 Caching

| Resource | Caching Recommendation |
| --- | --- |
| Machine list (GET /machines) | Cache for at least 1 hour. Machines change infrequently. |
| Telemetry data | Use storedAt timestamp to avoid re-processing duplicate records. |
| Error data | Cache with short TTL; errors can be time-critical. |

## 9.4 Error Handling

# Python: session with automatic retry and backoff

from requests.adapters import HTTPAdapter

from requests.packages.urllib3.util.retry import Retry

def create_session():

session = requests.Session()

retry = Retry(

total=3, backoff_factor=1,

status_forcelist=[429, 500, 502, 503, 504]

)

session.mount("https://", HTTPAdapter(max_retries=retry))

return session

## 9.5 Version Selection Strategy

| Scenario | Recommendation |
| --- | --- |
| New integration | Start with v2.0. It is the recommended version for all new projects. |
| Existing v1.1 integration | Migrate to v2.0 during the next maintenance window (see Section 6.6). |
| Legacy systems | Continue on v1.1 until an official deprecation notice is issued. |

## 9.6 Data Processing – v2.0 DDI Parser

# Python: parse DDI values with unit conversion

def parse_ddi_values(boom_data):

ddi_map = {}

for item in boom_data.get("values", []):

ddi, value = item["ddi"], item["value"]

if ddi in [6, 7]:      ddi_map[ddi] = value / 1000   # → l/ha

elif ddi == 58382:     ddi_map[ddi] = value / 10     # → %

elif ddi == 271:       ddi_map[ddi] = value          # m²

else:                  ddi_map[ddi] = value

return ddi_map

## 9.7 GeoJSON Processing

// JavaScript: process telemetry, auto-detect v1.1 vs v2.0

function processTelemetry(data) {

data.geoJson.features.forEach(feature => {

const coords = feature.geometry.coordinates; // [lon, lat]

const props  = feature.properties;

const position = {

longitude: coords[0], latitude: coords[1],

timestamp: props.recordedAt

};

if (props.booms?.[0]?.designator) {

processV2Booms(props.booms);   // v2.0

} else if (props.booms?.[0]?.boom !== undefined) {

processV1Booms(props.booms);   // v1.1

}

});

}

# 10. Support & Contact

For technical questions, access requests, or issues with the HorschConnect Telematics API:

Technical Support:  portal@horsch.com

API Documentation:  https://telematics-doc.horsch.com/en/api

Swagger / OpenAPI:  https://telematics-api.horsch.com/swagger/index.html

Telematics Portal:  https://telematics.horsch.com

Document: Maintained by HORSCH Maschinen SE &Co. KG • Last Updated: 2026-05-26 • Document Version: 1.2